TU/ CODL

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (AUTUMN) 2019

DRE 202: ENERGY EFFICIENCY IN THERMAL UTILITIES

Time:3 Hours				               Total Marks:70

The figures in the right-hand margin indicate marks for the individual question

*****************************************************

1. Fill in the blanks with appropriate word/phrases:                   1×10=10

- The basic cycle of operation of a coal based power plant is known as _________________________

- Critical point in water vapour cycle is a thermodynamic state given by a pressure above ________________ bar.

- Major part of the heat transfer from the flue gas to outer surface of economiser tube is taken place by ____________ mode.

- Critical radius of thermal insulation depends upon thermal conductivity of the insulation and ______________.

- Access air of combustion is defined as _____________.

- Fiberglass, mineral wool and polyurethane foam are types of ______________________ materials.

- Cogeneration power plant is defined as _____________.

- kJ/kg is the unit of specific enthalpy, whereas __________ is the unit of specific entropy.

- LMTD stands for _____________________________.

- Parallel-flow and counter-flow, Finned and Unfinned etc are types of _____________________.

2. Discuss the major criteria of classification of coal. Also discuss

the desirable properties of coal with the help of suitable

examples.			     	                                         5+5= 10

3. With the help of neat sketch describe the functions of different

boiler mountings. Also state briefly the periodic maintenance

requirements of a boiler.                                                         5+5= 10

P.T.O.

4. Justify the need of waste heat recovery in a thermal utility. Also

state four different practices of waste heat recovery known to

you. 	       		                                                           6+4=10

5. With the help of neat sketch explain the working of a Shell and

tube heat exchanger. Also explain the method used to assess its

performance. 	                                                                      5+5= 10

6. How much thermal energy is required for 1 tonne of water at

25°C to convert into dry steam at 5 bar in one hour? Also

estimate the hourly rate of coal consumption for the above.

Assume hf = 640.1 kJ/kg and hfg =2107.4 kJ/kg at 5 bar; CV of

coal = 30 MJ/kg and overall efficiency of boiler=95%.				                                                                                            5+5= 10

7. Answer the following.                                                                 5x2=10

- Determine the minimum flow rate of hot water (80 oC) in a Counter-flow heat exchanger to raise temperature of cold water (flow rate=1 litre/s) by 25°C. The value of specific heat capacity of water is approximately 4.2 J/g°C at room temperature.

- Determine the stoichiometric air-to-fuel ratio of a gaseous fuel having 70% propane (C3H8) and 30% butane (C4H10) by volume.

***