TU/ CODL

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (AUTUMN ) 2019

DRE 104: WIND AND HYDRO ENERGY

Time: 3 Hours                                  Total Marks: 70

The figures in the right-hand margin indicate marks for the individual question.

****************************************************

1. Choose the correct answer                                           2×10=20

a. Turbines blades have ____ type cross section to extract

energy from wind

i. Aero foil				ii. Elliptical

iii. Rectangular			iv. Any type

b. Which nation leads the world in installed wind energy

capacity?

i. China			            ii. USA

iii. India				iv. Netherlands

c. Which of these is NOT a part of a modern wind turbine?

i. Gearbox				ii. Nacelle

iii. Compressor			iv. Yaw drive

d. What is the main source for the formation of wind?

i. Uneven land			ii. Solar Radiation

iii. Vegetation			iv. Seasons

e. Why HAWT are more popular over VAWT?

i. Easy to manufacture 	            ii. Efficiency is high

iii. Number of blade is less	            iv. Cost of generation is

low

f. The value of maximum power coefficient (CPmax) is:

i. 0.593				ii. 0.898

iii. 0.321				iv. 0.333

P.T.O.

g. Pitch regulation is related to:

i. Yaw mechanism			ii. blade setting angle

iii. Wind direction		            iv. Power output

h. What will be the power available if overall efficiency of plant

is 80%, flow rate is 4.42 m3/s and head 400 m?

i. 6.52 MW				ii. 8.18 MW

iii. 11.255 MW			iv. 13.875 MW

I. What is the basic requirement of hydroelectric power station?

i. Reservoir				ii. Turbine

iii. Power house		            iv. Penstock

j. Which plants supply the peak load for the base power plants?

i. Mini hydel plants			ii. Pump storage power

plants

iii. Low head plants			iv. Run-off-river power

plants

2. Answer the following shortly:                                   3x5=15

- Explain time distribution and frequency distribution of wind speed for a site.

- What are the main differences between wind mill and wind electric generator?

- Draw a schematic diagram of a wind electric energy conversion system and discuss its various components.

- Discuss the various factors to be considered while selecting a site for a hydroelectric plant.

- What are the uses of hydrographs and flow duration curves?

3. Answer the following:                                          5+3+3+4=15

- Define Betz limit and derive the value for CP (max).

- Write down the assumptions used in axial momentum theory and blade element theory.

- Write down the different steps to be followed for designing a wind turbine.

- A wind generator with a 10m- diameter blade span has a cut-in wind speed (minimum speed for power generation) of 5 m/s, at which velocity; the turbine generates 3 kW electric powers. Determine the efficiency of wind-turbine generator set.

4.  Answer the following:                                         5+5+5+5=20

- What are the various factors considered in designing a hydroelectric power plant?

- Draw a schematic diagram of a hydropower plant. Explain in brief the various components of this power plant.

- The design head at China’s Three Gorges Dam, the largest hydropower project in the world, is 80.6 m and rated power output of each of the 32 turbine generator units is 700 MW. What is the design ﬂow through each turbine, if the efficiency of each of the turbine-generator unit at full power output is 93.2%?

- A factory is located near a waterfall where the usable head for power generation is 25 m. The factory requires continuous power of 400 kW throughout the year. The river flow in a year are (i) 10 m3/s for 4 months, (ii) 6 m3/s for 2 months and (iii) 1.5 m3/s for 6 months respectively. If the site is developed as a run-of-river type of plant, without storage, determine the standby capacity to be provided. Assume the overall efficiency of the plant is 80%.  If a reservoir is arranged in the upstream, will any standby unit be necessary? What will be the excess power available to the grid?

***