TU/CODL

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (SPRING) 2019

DRE 104: WIND AND HYDRO ENERGY

Time: 3 Hours                                                  Total Marks: 70

The figures in the right-hand margin indicate marks for the individual question.

*******************************************************

- Choose the right answer                                                                2x10= 20

- Which of the following energy source is an indirect method of Solar energy utilization?

(i) Wind Energy			(ii) Geothermal Energy

(iii) Nuclear energy	 	(iv) Tidal Energy

- The instrument preferred in the measurement of highly fluctuating velocities in air flows is

(i) Hot Wire Anemometer		(ii) Propeller type

Anemometer

(iii) Pitot Static Tube		(iv) Three cup

Anemometer

- Where is the largest Wind Farm located in India?

(i) Jaisalmer, Rajasthan		(ii) Muppandal, Tamil

Nadu

(iii) Vaspet, Maharashtra		(iv) Chakala,

Maharashtra

- A Kaplan turbine is

(i) a high head mixed flow turbine	(ii) an impulse turbine,

inward flow type

(iii) a reaction turbine,                       (iv) low head axial

outward flow type	                       flow turbine

- The coefficient of performance of a wind turbine for maximum power output is

(i) 13/18 				(ii) 11/17

(iii) 5/8				(iv) 16/27

P.T.O.

- In hydro power plants

- Initial cost is high and operating cost is low

- Initial cost as well as operating costs are high

(iii) Initial cost is low and operating cost is high

(iv) Initial cost as well as operating costs are low

- The share of hydro in India’s electricity installed capacity is

(i) 13%				(ii) 26%

(iii) 23%				(iv) 45%

- Cut-out velocity in a wind power machine is

- The maximum wind speed at which a wind turbine designed to stop operation

- The maximum wind speed at which a wind turbine is designed to come into operation

- The wind speed at which the wind turbine gives optimum power

- The velocity of the rotor

- About how much power is available when wind speed is 6m/s compared to when it is 3 m/s?

(i) About the same           (ii) About twice as much

(iii) About four times      (iv) About eight time as much as

as much as much

- The term Darrious rotor and Savonius rotor are related to

(i) Pico hydropower          (ii) Vertical axis wind turbine

(iii) Horizontal axis wind (iv) Battery charging systems

turbine

- Answer the following questions shortly: -                                   3×5=15

- Write down the various assumptions considered in the Betz limit calculation.

- Write down the different steps to be followed for designing a wind turbine.

- Discuss the various factors which are to be considered while selecting a site for a wind farm.

- What are the advantages of small hydro power plant over large hydro power plants?

- Define:  unit speed, unit power and unit discharge.

- (a)  Define Betz limit and derive the value for CP (max).

(b) A wind data acquisition system measures wind speed 6 m/s for

19 times, 7 m/s for 54 times, and 8m/s for 42 times during a

given period. Find out the average wind speed and standard

deviation of the measured wind speeds.

(c) Evaluate the probability density function of Weibull

distribution for c = 1m/s for wind speeds of 0.2, 0.4, 0.8, 1.2, 1.6,

and 2.0 m/s at k = 3.2. Plot the probability density function

versus wind speed profile.

5+5+5=15

- (a) Discuss the applications of hydroelectric power plant for producing power in commercial scale. What are the main characteristics of a good plant site?

- What do you mean by the following-

- hydrograph          (ii) Flow duration curve

(iii)   power duration curve?

- A hydroelectric station is under mean head of 30 meters. The reservoir employed has a catchment area 400 km2. The average rainfall in this area is 125 cm per annum. Determine the capacity of the system (MW) for which it should be designed. Assume that 30 percent of rainfall is lost due to evaporation etc., 5 per cent of the head lost in penstock, turbine efficiency as 85 per cent, alternator efficiency as 90 per cent.

- The design head at China’s Three Gorges Dam, the largest hydropower project in the world, is 80.6 m and rated power output of each of the 32 turbine generator units is 700 MW. What is the design ﬂow through each turbine, if the efficiency of each of the turbine-generator unit at full power output is 93.2%?

5+5+5+5=20

********