TU/CDOE

TEZPUR UNIVERSITY

SEMESTER END EXAMINATION (SPRING) 2021

DRE 201: ENERGY MANAGEMENT AND AUDITING

Full Marks: 70	                                    Time: 3 hours

The figures in the right-hand margin indicate marks

for the individual questions

---------------------------------------------------------------------------------

1. Choose the correct answer:                                                           2×10=20

- Energy management involves-

(i) combination of technical and managerial skills

(ii) managerial skills

(iii) technical skills

(iv) energy audit skills

- Which of the following is least important in a good energy information system?

(i) data accuracy        	 	   (ii) data relevance

(iii) data volume          		   (iv) data validation

- The ratio of energy consumption to corresponding production is called:

(i) specific energy consumption  (ii) Energy intensity

(iii) production energy ratio        (iv) specific production ratio

- For an investment which has fluctuating annual savings over its project life, which of the following financial techniques is the best?

(i) Simple payback period 	     (ii) ROI

(iii) NPV		                    (iv) IRR

- The central nodal agency for coordinating and implementing the energy conservation activities in India under the EC Act, 2001 is:

(i) Bureau of Indian Standards

(ii) Bureau of Energy Efficiency

(iii) Bureau of Energy and Environment

(iv) Bureau of Energy Conservation

- The major loss in a fuel fired boiler is:

(i) Radiation loss		       (ii) Flue gas loss

(iii) Un-burnt carbon loss	       (iv) Blow down loss

P.T.O.

- A three-phase induction motor is drawing 12 amps at 440 volts. If the operating power factor of the motor is 0.85, then the power drawn by the motor is

(i) 7.8 kW			  (ii) 4.5 kW

(iii) 5.3 kW			  (iv) 23.4 kW

- ESCO stands for

(i) Energy supply company	   (ii) Energy service company

(iii) Energy standards company (iv) Energy sourcing

company

- Energy consumed for the period is described by the following equation (C= fixed and M is variable energy consumption)

(i) C+M ×Production		     (ii) C × M + Production

(iii)  (C+M) × Production 	     (iv) C × M × Production

- CPM of a project management refers to

(i) critical periodic management (ii) critical path method

(iii) cost project management	     (iv) critical project

monitoring

2. Answer the following:                                                              5+5+4+6=20

- Explain the difference between energy conservation and energy efficiency and state one example where energy costs are reduced but energy consumption goes up.

- Explain five important features of Energy Conservation Act, 2001.

- Explain briefly:

- guaranteed saving performance contract

- shared savings performance contract.

- An investment of Rs. 1.0 lakh is made for a variable speed drive at the beginning of the year, which is also the date of operation. Savings expected over 4 years are Rs. 30,000, Rs. 30,000, Rs. 40,000 and Rs 45,000 respectively. Find out the Net Present Value (NPV) at the end of the 4th year, if the discount rate is 12%. Also calculate the IRR.

3. Answer the following:                                                              3+3+3+6=15

- What are the different types of energy auditing?

- What are the similarities and dissimilarities between energy auditing and financial auditing?

- Why the economic analysis is so important in energy management programme?

P.T.O.

- A plant is using 4 tons/day of coal to generate steam. The calorific value of the coal is 4000 kcal/kg. The cost of coal is Rs. 2000/tone. The plant substitute coal with rice husks, as a boiler fuel, which has a calorific value of 3000 kcal/kg and cost Rs. 700/tone. Calculate the annual cost savings at 300 days of operation, assuming the boiler efficiency decreases from 78% on coal to 72% on rice husks.

4. Answer the following:                                                                   5+5+5=15

- What is reactive power, apparent power and active power?

- The fuel input to a furnace is 800 kg/h. The material feed rate is 10 tones/h. The temperature attained in the furnace is 1340 0C. The specific heat of the material is 0.12 kCal/kgK. The ambient temperature is 30 0C and CV of the fuel is 10000 kCal/kg. Calculate the furnace efficiency.

- A municipal corporation has 30,000 numbers of 250 Watts HPMV lamps. The annual operation hours of these lamps are 3600 hours. Calculate the annual energy cost, if the electricity tariff is Rs 4.75/kWh. Estimate the demand cost, if the present demand cost is Rs 150/KVA; assume Power factor is 0.7 for the lighting load. Discuss at least one energy savings opportunity in this lighting system, with investment and simple payback period.

***